|  | - Mainsail - | 3000112 Bernard |  |
| --- | --- | --- | --- |
| M0 | Sail Layout | See picture ‘Layout’ |  |
| M1 | Tack Cut Back (Aft face of Mast to center of tack Pin - A) | 1.5 inch Ring flush with corner. Picture 1 | HELP |
| M2 | Tack Cut Up (Top of Boom to Center of tack Pin - B) | 0.5 inch ( pic 1) | HELP |
| M3 | Clew Cut Up (Top of Boom to Center of Tack Pin - X) | 0.5 inch. Ring is flush with sail (Picure 2) | HELP |
| M4 | Clew type (slug, outhaul car or ring with strap) | Ring with strap (pic 2) | HELP |
| M5 | Sunbrella clew cover for Furling Mains | na | HELP |
| M6 | Other Offsets |  | HELP |
| M7 | Headboard | Yes | HELP |
| M8 | Leech Line | yes |  |
| M9 | Foot Line | No |  |
| M10 | Tack Ring Type | Ring + strap. | HELP |
| M11 | Tack angle | 88.5 | HELP |
| M12 | Jack Line | No | HELP |
| M13 | Mast Bend (Luff Round) | Std | HELP |
| M14 | Fractional Rig (Yes, No) | No | HELP |
| M15 | Mast - Keel or Deck Stepped? | Keel |  |
| M16 | Number of Reefs | 3 Reefs | HELP |
| M17 | Intermediate Reefing Diamond Eyes (Yes or No) | No | HELP |
| M18 | Foot Reefing Grommets (next to boom) | N/A | HELP |
| M19 | Reefing Ring Type | Press ring + strap.(Picture 3) | HELP |
| M20 | Luff Length (Max Hoist - P) | 43’6’’ Max (Net Finished size day of Delivery) | HELP |
| M21 | Foot Length (Max Foot - E) | 13’3’’ Max (Net Finished size day of Delivery) | HELP |
| M22 | Leech Length (straight line head to clew) | Sailmaker | HELP |
| M23 | Bimini to Clear (yes or no) | N/A | HELP |
| M24 | Foot Round | Sailmaker |  |
| M25 | Aspect Ratio | Sailmaker | HELP |
| M26 | Vessel Backstay Measurement | >15’( twin backstays) | HELP |
| M27 | Luff Slides (Part Number) | 10x Plastic hourglass-shaped spacers see picture4 + Diagram. Plastic shape: height:20mm. Diameter:12mm. Hole diameter:4mm. For use on harken ball bearing slides on mast. | HELP |
| M28 | Foot Slides (Part Number) | Loose foot | HELP |
| M29 | Roach Type | Cruise | HELP |
| M30 | Roach Factor | 1.1 | HELP |
| M31 | MGM | Sailmaker | HELP |
| M32 | MGU | Sailmaker | HELP |
| M33 | Batten Type (Partial or Full) | 4 full | HELP |
| M34 | Include Battens | No |  |
| M35 | Install Nylon Batten Holders | Holders being sent to loft | HELP |
| M36 | Intermediate Sliders | 2 between Each Batten | HELP |
| M37 | Cloth Weight | 9.3 |  |
| M38 | Cut Type | Cross |  |
| M39 | Sail Area | Sailmaker |  |
| M40 | Cunningham | No | HELP |
| M41 | Telltails | Yes | HELP |
| M42 | Triple Stitch | Yes |  |
| M43 | Sail Logo and Color(red, blue or black) |  | HELP |
| M44 | Sail Logo Artwork | see picture 5 |  |
| M45 | Sail Numbers and Color |  | HELP |
| M46 | Draft Stripe and Color(red, blue or black) | Yes (Black) | HELP |
| M47 | Overhead Leech Line | No |  |
| M48 | Specialty Sail | No | HELP |
| M49 | Boom Cover | No | HELP |
| M50 | Lazy Bag | Yes, see below. | HELP |

Picture A: Mainsail layout

Picture 1: Tack cut M1+ M2

Picture 2: Clew cut up

Picture 3 : Press ring+ straps for reefing points (both ends  of sail)

Plastic hourglass-shaped spacers: 20mm high, 12mm diameter and 4mm hole for clevis pin.

Picture 4-a

Picture 4-b

Lugs are set back 1 inch from sail, measured from centerhole to sail edge.  Double webbing 1 inch is used to attach lug to grommet on the sail.

Picture 5: Nauticat LOGO

Lazybag Layout form:

- Mast circumference: 36’’

- Distance from end of boom to the mast: 13.94

- Maximum height of lazybag: 4’4’’

- Circumference of lazybag end: 3’

- Circumference of lazybag ¼: 4’

- Circumference of lazybag ½: 4’6’’

- Circumference of lazybag ¾: 5’6’’

- Distance from end of bag to 1st eyelet: 3’

- Distance from first to second eyelet: 2’9’’

- Distance from 2nd to 3rd eyelet: 2’ 9’’

Distance from 3rd to 4th eyelet: 2’9’’ ( there are 4 eyelets on my cover)

- Bottom is a 10 mm boltrope.

- Boltrope diameter: 10mm

- Bottom of bag is closed.

Other remarks: Present boltrope is 11’ long so that reefing can occur.  Boltrope starts one feet behind mast ( and stops 2 feet before end of boom).  The bag has some more room at the base of the mast as the battcar systems stacks on top of another and this creates more height.  Let me know if this is a possible way to make the bag or if you suggest making it symmetrical.

Top of bag is a nylon zipper.

There is an opening at the sides of lazybag for the reefing lines to exit the bag and attach the reefing lines to the boom.  That opening is stiched in the middle.  See picture A.

Lazybag openings for reefing lines ( both sides, reinforced)

Actual lazybag shape: Enlargement at base of mast for battcar stakings