|  | - Mainsail - | 3000414 Luke |  |
| --- | --- | --- | --- |
| M0 | Sail Layout | Not Yet Posted |  |
| M1 | Tack Cut Back (Aft face of Mast to center of tack Pin - A) | 45mm | HELP |
| M2 | Tack Cut Up (Top of Boom to Center of tack Pin - B) | 40mm | HELP |
| M3 | Clew Cut Up (Top of Boom to Center of Tack Pin - X) | 35mm | HELP |
| M4 | Clew type (slug, outhaul car or ring with strap) | Ring & Strap | HELP |
| M5 | Sunbrella clew cover for Furling Mains | na | HELP |
| M6 | Other Offsets | A = 45 mm (Reefing hook cut back) B = 40mm (reefing hook cut up) D= 140mm boom groove cut back | HELP |
| M7 | Headboard | Yes | HELP |
| M8 | Leech Line | yes |  |
| M9 | Foot Line | No |  |
| M10 | Tack Ring Type | Press Ring | HELP |
| M11 | Tack angle | 88.5 | HELP |
| M12 | Jack Line | No | HELP |
| M13 | Mast Bend (Luff Round) | Std | HELP |
| M14 | Fractional Rig (Yes, No) | Yes | HELP |
| M15 | Mast - Keel or Deck Stepped? | Deck Stepped |  |
| M16 | Number of Reefs | 2 Reefs | HELP |
| M17 | Intermediate Reefing Diamond Eyes (Yes or No) | Yes | HELP |
| M18 | Foot Reefing Grommets (next to boom) | N/A | HELP |
| M19 | Reefing Ring Type | Press ring | HELP |
| M20 | Luff Length (Max Hoist - P) | 8530mm Max (Net Finished size day of Delivery) | HE8LP |
| M21 | Foot Length (Max Foot - E) | 3110mm Max (Net Finished size day of Delivery) | HELP |
| M22 | Leech Length (straight line head to clew) | Sailmaker | HELP |
| M23 | Bimini to Clear (yes or no) | N/A | HELP |
| M24 | Foot Round | Sailmaker |  |
| M25 | Aspect Ratio | Sailmaker | HELP |
| M26 | Vessel Backstay Measurement | 3800mm | HELP |
| M27 | Luff Slides (Part Number) | ??? (TBA) | HELP |
| M28 | Foot Slides (Part Number) | Fixed foot bolt rope | HELP |
| M29 | Roach Type | Racer/Cruiser | HELP |
| M30 | Roach Factor | 1.09 | HELP |
| M31 | MGM | Sailmaker | HELP |
| M32 | MGU | Sailmaker | HELP |
| M33 | Batten Type (Partial or Full) | 4 partial | HELP |
| M34 | Include Battens | Yes |  |
| M35 | Install Nylon Batten Holders | N/A | HELP |
| M36 | Intermediate Sliders | N/A | HELP |
| M37 | Cloth Weight | 5.93 |  |
| M38 | Cut Type | Cross |  |
| M39 | Sail Area | Sailmaker |  |
| M40 | Cunningham | No | HELP |
| M41 | Telltails | Yes | HELP |
| M42 | Triple Stitch | Yes |  |
| M43 | Sail Logo and Color(red, blue or black) | No | HELP |
| M44 | Sail Logo Artwork | N/A |  |
| M45 | Sail Numbers and Color | SIN711 (Red colour) | HELP |
| M46 | Draft Stripe and Color(red, blue or black) | Yes (Black) | HELP |
| M47 | Overhead Leech Line | No |  |
| M48 | Specialty Sail | No | HELP |
| M49 | Boom Cover | Yes (Red) | HELP |
| M50 | Lazy Bag | No | HELP |

A = 45 mm
B = 40 mm
C = Flexible reefing horns (see photo)
D = 140 mm
E = 310 mm
F = 
Backstay = 3800 mm

A = 320 mm
B = 
C = 420 mm
D = 3260 mm
E = 680 mm
F = 900 mm
G = 1050 mm
H = 1020 mm
I = 980 mm
J1 = 950 mm
J2 = (at end of boom.  No issue for fasteners)

Old Mainsail Measurements

9000mm

8320 mm (not under sailing load)

3020 mm (not under sailing load)

Factory Specifications Mainsail Measurements

9120 mm

8500 mm

3200 mm

Clew attachment